Unit 4 Transport Layer

Objectives

The transport layer is responsible for process-to-process delivery of the entire message.

A process is an application program running on a host. Whereas the network layer

oversees source-to-destination delivery of individual packets, it does not recognize any

relationship between those packets. It treats each one independently, as though each

piece belonged to a separate message, whether or not it does. The transport layer, on the

other hand, ensures that the whole message arrives intact and in order, overseeing both

error control and flow control at the source-to-destination level.

The transport layer is responsible for the delivery

of a message from one process to another.

Computers often run several programs at the same time. For this reason, sourceto-

destination delivery means delivery not only from one computer to the next but also

from a specific process on one computer to a specific process on the other. The transport

layer header must therefore include a type of address called a service-point address in

the OSI model and port number or port addresses in the Internet and TCP/IP protocol

suite.

A transport layer protocol can be either connectionless or connection-oriented.

A connectionless transport layer treats each segment as an independent packet and

delivers it to the transport layer at the destination machine. A connection-oriented

transport layer makes a connection with the transport layer at the destination machine

first before delivering the packets. After all the data is transferred, the connection is

terminated.

In the transport layer, a message is normally divided into transmittable segments. A

connectionless protocol, such as UDP, treats each segment separately. A connectionoriented

protocol, such as TCP and SCTP, creates a relationship between the segments

using sequence numbers.

Like the data link layer, the transport layer may be responsible for flow and error

control. However, flow and error control at this layer is performed end to end rather

than across a single link. We will see that one of the protocols discussed in this part of the book, UDP, is not involved in flow or error controL On the other hand, the other two

protocols, TCP and SCTP, use sliding windows for flow control and an acknowledgment

system for error control.

End-to-end delivery:

The transport layer transmits the entire message to the destination. Therefore, it ensures the end-to-end delivery of an entire message from a source to the destination.

Reliable delivery:

The transport layer provides reliability services by retransmitting the lost and damaged packets.

Flow Control

Flow control is used to prevent the sender from overwhelming the receiver. If the receiver is overloaded with too much data, then the receiver discards the packets and asking for the retransmission of packets. This increases network congestion and thus, reducing the system performance. The transport layer is responsible for flow control. It uses the sliding window protocol that makes the data transmission more efficient as well as it controls the flow of data so that the receiver does not become overwhelmed. Sliding window protocol is byte oriented rather than frame oriented.

Addressing

According to the layered model, the transport layer interacts with the functions of the session layer. Many protocols combine session, presentation, and application layer protocols into a single layer known as the application layer. In these cases, delivery to the session layer means the delivery to the application layer. Data generated by an application on one machine must be transmitted to the correct application on another machine. In this case, addressing is provided by the transport layer.

The transport layer provides the user address which is specified as a station or port. The port variable represents a particular TS user of a specified station known as a Transport Service access point (TSAP). Each station has only one transport entity.

The transport layer protocols need to know which upper-layer protocols are communicating.

Multiplexing

Whenever the bandwidth of a medium linking two devices is greater than the bandwidth

needs of the devices, the link can be shared. Multiplexing is the set of techniques

that allows the simultaneous transmission of multiple signals across a single data link.

As data and telecommunications use increases, so does traffic. We can accommodate

this increase by continuing to add individual links each time a new channel is needed;

or we can install higher-bandwidth links and use each to carry multiple signals. As

described in Chapter 7, today's technology includes high-bandwidth media such as

optical fiber and terrestrial and satellite microwaves. Each has a bandwidth far in excess

of that needed for the average transmission signal. If the bandwidth of a link is greater

than the bandwidth needs of the devices connected to it, the bandwidth is wasted. An

efficient system maximizes the utilization of all resources; bandwidth is one of the

most precious resources we have in data communications.

In a multiplexed system, n lines share the bandwidth of one link. Figure 6.1 shows

the basic format of a multiplexed system. The lines on the left direct their transmission

streams to a multiplexer (MUX), which combines them into a single stream (many-toone).

At the receiving end, that stream is fed into a demultiplexer (DEMUX), which

separates the stream back into its component transmissions (one-to-many) and

directs them to their corresponding lines. In the figure, the word link refers to the

physical path. The word channel refers to the portion of a link that carries a transmission

between a given pair of lines. One link can have many (n) channels.

There are three basic multiplexing techniques: frequency-division multiplexing,

wavelength-division multiplexing, and time-division multiplexing. The first two are

techniques designed for analog signals, the third, for digital signals (see Figure 6.2).

Frequency-Division Multiplexing

Frequency-division multiplexing (FDM) is an analog technique that can be applied

when the bandwidth of a link (in hertz) is greater than the combined bandwidths of

the signals to be transmitted. In FOM, signals generated by each sending device modulate

different carrier frequencies. These modulated signals are then combined into a single

composite signal that can be transported by the link. Carrier frequencies are separated by

sufficient bandwidth to accommodate the modulated signal. These bandwidth ranges are

the channels through which the various signals travel. Channels can be separated by strips of unused bandwidth-guard bands-to prevent signals from overlapping. In

addition, carrier frequencies must not interfere with the original data frequencies.

Multiplexing Process

Figure 6.4 is a conceptual illustration of the multiplexing process. Each source generates

a signal of a similar frequency range. Inside the multiplexer, these similar signals

modulates different carrier frequencies (/1,12, and h). The resulting modulated signals

are then combined into a single composite signal that is sent out over a media link that

has enough bandwidth to accommodate it.

Demultiplexing Process

The demultiplexer uses a series of filters to decompose the multiplexed signal into its

constituent component signals. The individual signals are then passed to a demodulator

that separates them from their carriers and passes them to the output lines. Figure 6.5 is

a conceptual illustration of demultiplexing process.

Wavelength-Division Multiplexing

Wavelength-division multiplexing (WDM) is designed to use the high-data-rate

capability of fiber-optic cable. The optical fiber data rate is higher than the data rate of

metallic transmission cable. Using a fiber-optic cable for one single line waste WDM is conceptually the same as FDM, except that the multiplexing and demultiplexing

involve optical signals transmitted through fiber-optic channels. The idea is the

same: We are combining different signals of different frequencies. The difference is

that the frequencies are very high.

Figure 6.10 gives a conceptual view of a WDM multiplexer and demultiplexer.

Very narrow bands of light from different sources are combined to make a wider band

of light. At the receiver, the signals are separated by the demultiplexer.

The Time Division Multiplexing (TDM) is a digital procedure. Here, each sender is given the entire possession of the whole bandwidth of the channel for a fixed duration of time. After this, the control is moved to the next sender, and the process continues on a round-robin basis.

An example of TDM is the television broadcast. In a television serial, generally, a 10 minutes' serial is followed by a 5 minutes' advertisement. The time in which the serial is being broadcasted, the total frequency is dedicated to the serial.

Variants

The following variants on TDM are as follows −

ATDM (Asynchronous Time Division Multiplexing): Multiplexing in which the data is transmitted asynchronously.

STDM (Statistical Time Division Multiplexing): A multiplexing method that polls nodes and immediately skips any nodes with nothing to send.

STM (Synchronous Transfer Node): Designer for use in BISDN (broadband ISDN) and also supported in the SONET (Asynchronous Optical Network) architecture.

Types

There are two types of TDM which are as follows −

Synchronous TDM

Synchronous TDM is known as synchronous and is essential because, each time slot is pre-assigned to a constant source. The time slots are sent irrespective of whether the sources have a few records to share or not.

TDM devices can manage the source of various data rates. This is completed by authorising fewer slots per cycle to the passive input devices than the rapid device.

Both multiplexing and demultiplexing operations for synchronous TDM, are demonstrated in the figure given below.

Statistical TDM

One disadvantage of the TDM method is that some of the time slots in the frame are wasted. A specific terminal has no information to send to a particular instant of time and will share an unfilled time slot. It is also called asynchronous TDM or intelligent TDM.

UDP

User Datagram Protocol (UDP) is a Transport Layer protocol. UDP is a part of the Internet Protocol suite, referred to as UDP/IP suite. Unlike TCP, it is an unreliable and connectionless protocol. So, there is no need to establish a connection prior to data transfer. The UDP helps to establish low-latency and loss-tolerating connections establish over the network.The UDP enables process to process communication.

Though Transmission Control Protocol (TCP) is the dominant transport layer protocol used with most of the Internet services; provides assured delivery, reliability, and much more but all these services cost us additional overhead and latency. Here, UDP comes into the picture. For real-time services like computer gaming, voice or video communication, live conferences; we need UDP. Since high performance is needed, UDP permits packets to be dropped instead of processing delayed packets. There is no error checking in UDP, so it also saves bandwidth.

User Datagram Protocol (UDP) is more efficient in terms of both latency and bandwidth.

UDP Header –

UDP header is an 8-bytes fixed and simple header, while for TCP it may vary from 20 bytes to 60 bytes. The first 8 Bytes contains all necessary header information and the remaining part consist of data. UDP port number fields are each 16 bits long, therefore the range for port numbers is defined from 0 to 65535; port number 0 is reserved. Port numbers help to distinguish different user requests or processes.

User Datagram

UDP packets, called user datagrams, have a fixed-size header of 8 bytes. Figure 23.9

shows the format of a user datagram.

The fields are as follows:

o Source port number. This is the port number used by the process running on the

source host. It is 16 bits long, which means that the port number can range from 0 to

65,535. If the source host is the client (a client sending a request), the port number, in

most cases, is an ephemeral port number requested by the process and chosen by the

UDP software running on the source host. If the source host is the server (a server

sending a response), the port number, in most cases, is a well-known port number.

Destination port number. This is the port number used by the process running on

the destination host. It is also 16 bits long. If the destination host is the server (a

client sending a request), the port number, in most cases, is a well-known port

number. If the destination host is the client (a server sending a response), the port

number, in most cases, is an ephemeral port number. In this case, the server copies

the ephemeral port number it has received in the request packet.

o Length. This is a 16-bit field that defines the total length of the user datagram,

header plus data. The 16 bits can define a total length of 0 to 65,535 bytes. However,

the total length needs to be much less because a UDP user datagram is stored

in an IP datagram with a total length of 65,535 bytes.

The length field in a UDP user datagram is actually not necessary. A user

datagram is encapsulated in an IP datagram. There is a field in the IP datagram

that defines the total length. There is another field in the IP datagram that defines

the length of the header. So if we subtract the value of the second field from the

first, we can deduce the length of a UDP datagram that is encapsulated in an

IP datagram.

However, the designers of the UDP protocol felt that it was more efficient for the

destination UDP to calculate the length of the data from the information provided

in the UDP user datagram rather than ask the IP software to supply this information.

We should remember that when the IP software delivers the UDP user datagram to

the UDP layer, it has already dropped the IP header.

o Checksum. This field is used to detect errors over the entire user datagram (header

plus data). The checksum is discussed next.

Checksum

We have already talked about the concept of the checksum and the way it is calculated

in Chapter 10. We have also shown how to calculate the checksum for the IP and ICMP

packet. We now show how this is done for UDP.The UDP checksum calculation is different from the one for IP and ICMP. Here

the checksum includes three sections: a pseudoheader, the UDP header, and the data

coming from the application layer.

Notes – Unlike TCP, the Checksum calculation is not mandatory in UDP. No Error control or flow control is provided by UDP. Hence UDP depends on IP and ICMP for error reporting. Also UDP provides port numbers so that is can differentiate between users requests.

UDP Operation

UDP uses concepts common to the transport layer. These concepts will be discussed

here briefly, and then expanded in the next section on the TCP protocol.

Connectionless Services

As mentioned previously, UDP provides a connectionless service. This means that each

user datagram sent by UDP is an independent datagram. There is no relationship

between the different user datagrams even if they are coming from the same source process

and going to the same destination program. The user datagrams are not numbered.

Also, there is no connection establishment and no connection termination, as is the case

for TCP. This means that each user datagram can travel on a different path.

One of the ramifications of being connectionless is that the process that uses UDP

cannot send a stream of data to UDP and expect UDP to chop them into different

related user datagrams. Instead each request must be small enough to fit into one user

datagram. Only those processes sending short messages should use UDP.

Flow and Error Control

UDP is a very simple, unreliable transport protocol. There is no flow control and hence

no window mechanism. The receiver may overflow with incoming messages.

There is no error control mechanism in UDP except for the checksum. This means

that the sender does not know if a message has been lost or duplicated. When the receiver

detects an error through the checksum, the user datagram is silently discarded.

The lack of flow control and error control means that the process using UDP

should provide these mechanisms.

Encapsulation and Decapsulation

To send a message from one process to another, the UDP protocol encapsulates and

decapsulates messages in an IP datagram.

Queuing

We have talked about ports without discussing the actual implementation of them. In

UDP, queues are associated with ports

Use ofUDP

The following lists some uses of the UDP protocol:

o UDP is suitable for a process that requires simple request-response communication

with little concern for flow and error control. It is not usually used for a process

such as FrP that needs to send bulk data (see Chapter 26).

o UDP is suitable for a process with internal flow and error control mechanisms. For

example, the Trivial File Transfer Protocol (TFTP) process includes flow and error

control. It can easily use UDP.

o UDP is a suitable transport protocol for multicasting. Multicasting capability is

embedded in the UDP software but not in the TCP software.

o UDP is used for management processes such as SNMP (see Chapter 28).

o UDP is used for some route updating protocols such as Routing Information Protocol

(RIP)

Applications of UDP:

Used for simple request-response communication when the size of data is less and hence there is lesser concern about flow and error control.

It is a suitable protocol for multicasting as UDP supports packet switching.

UDP is used for some routing update protocols like RIP(Routing Information Protocol).

Normally used for real-time applications which can not tolerate uneven delays between sections of a received message.

Following implementations uses UDP as a transport layer protocol:

NTP (Network Time Protocol)

DNS (Domain Name Service)

BOOTP, DHCP.

NNP (Network News Protocol)

Quote of the day protocol

TFTP, RTSP, RIP.

The application layer can do some of the tasks through UDP-

Trace Route

Record Route

Timestamp

UDP takes a datagram from Network Layer, attaches its header, and sends it to the user. So, it works fast.

Actually, UDP is a null protocol if you remove the checksum field.

Reduce the requirement of computer resources.

When using the Multicast or Broadcast to transfer.

The transmission of Real-time packets, mainly in multimedia applications.

Connection-Oriented Service

Connection-Oriented Service is basically a technique that is typically used to transport and send data at session layer. The data streams or packets are transferred or delivered to receiver in a similar order in which they have seen transferred by sender. It is actually a data transfer method among two devices or computers in a different network, that is designed and developed after telephone system. Whenever a network implements this service, it sends or transfers data or message from sender or source to receiver or destination in correct order and manner.

TCP

The second transport layer protocol we discuss in this chapter is called Transmission

Control Protocol (TCP). TCP, like UDP, is a process-to-process (program-to-program)

protocol. TCP, therefore, like UDP, uses port numbers. Unlike UDP, TCP is a connectionoriented

protocol; it creates a virtual connection between two TCPs to send data. In

addition, TCP uses flow and error control mechanisms at the transport level.

In brief, TCP is called a connection-oriented, reliable transport protocol. It adds

connection-oriented and reliability features to the services of IP.

TCP Services

Before we discuss TCP in detail, let us explain the services offered by TCP to the processes

at the application layer.

Process-to-Process Communication

Like UDP, TCP provides process-to-process communication using port numbers. Table 23.2

lists some well-known port numbers used by TCP.

Stream Delivery Service

TCP, unlike UDP, is a stream-oriented protocol. In UDP, a process (an application program)

sends messages, with predefined boundaries, to UDP for delivery. UDP adds its

own header to each of these messages and delivers them to IP for transmission. Each

message from the process is calIed a user datagram and becomes, eventually, one IP

datagram. Neither IP nor UDP recognizes any relationship between the datagrams.

TCP, on the other hand, allows the sending process to deliver data as a stream of

bytes and allows the receiving process to obtain data as a stream of bytes. TCP creates

an environment in which the two processes seem to be connected by an imaginary "tube"

that carries their data across the Internet. This imaginary environment is depicted in

Figure 23.13. The sending process produces (writes to) the stream of bytes, and the

receiving process consumes (reads from) them.

Full-Duplex Communication

TCP offers full-duplex service, in which data can flow in both directions at the same time.

Each TCP then has a sending and receiving buffer, and segments move in both directions.

Connection-Oriented Service

TCP, unlike UDP, is a connection-oriented protocol. When a process at site A wants to

send and receive data from another process at site B, the following occurs:

1. The two TCPs establish a connection between them.

2. Data are exchanged in both directions.

3. The connection is terminated.

Note that this is a virtual connection, not a physical connection. The TCP segment is

encapsulated in an IP datagram and can be sent out of order, or lost, or corrupted, and then

resent. Each may use a different path to reach the destination. There is no physical connection.

TCP creates a stream-oriented environment in which it accepts the responsibility of delivering the bytes in order to the other site. The situation is similar to creating a bridge

that spans multiple islands and passing all the bytes from one island to another in one

single connection. We will discuss this feature later in the chapter.

Reliable Service

TCP is a reliable transport protocol. It uses an acknowledgment mechanism to check

the safe and sound arrival of data. We will discuss this feature further in the section on

error control.

TCP Features

To provide the services mentioned in the previous section, TCP has several features that

are briefly summarized in this section and discussed later in detail.

Numbering System

Although the TCP software keeps track of the segments being transmitted or received,

there is no field for a segment number value in the segment header. Instead, there are

two fields called the sequence number and the acknowledgment number. These two

fields refer to the byte number and not the segment number.

Byte Number TCP numbers all data bytes that are transmitted in a connection. Numbering

is independent in each direction. When TCP receives bytes of data from a process, it

stores them in the sending buffer and numbers them. The numbering does not necessarily

start from O. Instead, TCP generates a random number between 0 and 232 - 1 for the number

of the first byte. For example, if the random number happens to be 1057 and the total

data to be sent are 6000 bytes, the bytes are numbered from 1057 to 7056. We will see that

byte numbering is used for flow and error control.

Flow Control

TCP, unlike UDP, provides flow control. The receiver of the data controls the amount of

data that are to be sent by the sender. This is done to prevent the receiver from being overwhelmed

with data. The numbering system allows TCP to use a byte-oriented flow control.

Error Control

To provide reliable service, TCP implements an error control mechanism. Although

error control considers a segment as the unit of data for error detection (loss or corrupted

segments), error control is byte-oriented, as we will see later.

Congestion Control

TCP, unlike UDP, takes into account congestion in the network. The amount of data sent

by a sender is not only controlled by the receiver (flow control), but is also detennined

by the level of congestion in the network.

Segment

Before we discuss TCP in greater detail, let us discuss the TCP packets themselves. A

packet in TCP is called a segment.

The segment consists of a 20- to 60-byte header, followed by data from the application

program. The header is 20 bytes if there are no options and up to 60 bytes if it

contains options. We will discuss some of the header fields in this section. The meaning

and purpose of these will become clearer as we proceed through the chapter.

o Source port address. This is a 16-bit field that defines the port number of the

application program in the host that is sending the segment. This serves the same

purpose as the source port address in the UDP header.

o Destination port address. This is a 16-bit field that defines the port number of the

application program in the host that is receiving the segment. This serves the same

purpose as the destination port address in the UDP header.

o Sequence number. This 32-bit field defines the number assigned to the first byte of

data contained in this segment. As we said before, TCP is a stream transport protocol.

To ensure connectivity, each byte to be transmitted is numbered. The sequence number

tells the destination which byte in this sequence comprises the first byte in the segment.

During connection establishment, each party uses a random number generator to

create an initial sequence number (ISN), which is usually different in each direction.

o Acknowledgment number. This 32-bit field defines the byte number that the

receiver of the segment is expecting to receive from the other party. If the receiver of the segment has successfully received byte number x from the other party, it

defines x + I as the acknowledgment number. Acknowledgment and data can be

piggybacked together.

D Header length. This 4-bit field indicates the number of 4-byte words in the TCP

header. The length of the header can be between 20 and 60 bytes. Therefore, the

value of this field can be between 5 (5 x 4 =20) and 15 (15 x 4 =60).

D Reserved. This is a 6-bit field reserved for future use.

D Control. This field defines 6 different control bits or flags as shown in Figure 23.17.

One or more of these bits can be set at a time.

These bits enable flow control, connection establishment and termination, connection

abortion, and the mode of data transfer in TCP. A brief description of each bit is shown in

Table 23.3. We will discuss them further when we study the detailed operation of TCP

later in the chapter.

D Window size. This field defines the size of the window, in bytes, that the other

party must maintain. Note that the length of this field is 16 bits, which means that

the maximum size of the window is 65,535 bytes. This value is normally referred

to as the receiving window (rwnd) and is determined by the receiver. The sender

must obey the dictation of the receiver in this case.

D Checksum. This 16-bit field contains the checksum. The calculation of the checksum

for TCP follows the same procedure as the one described for UDP. However, the

inclusion of the checksum in the UDP datagram is optional, whereas the inclusion

of the checksum for TCP is mandatory. The same pseudoheader, serving the same purpose, is added to the segment. For the TCP pseudoheader, the value for the protocol

field is 6.

o Urgent pointer. This l6-bit field, which is valid only if the urgent flag is set, is

used when the segment contains urgent data. It defines the number that must be

added to the sequence number to obtain the number of the last urgent byte in the

data section of the segment. This will be discussed later in this chapter.

o Options. There can be up to 40 bytes of optional information in the TCP header.

We will not discuss these options here; please refer to the reference list for more

information.

A TCP Connection

TCP is connection-oriented. A connection-oriented transport protocol establishes a

virtual path between the source and destination. All the segments belonging to a message

are then sent over this virtual path. Using a single virtual pathway for the entire

message facilitates the acknowledgment process as well as retransmission of damaged

or lost frames. You may wonder how TCP, which uses the services of IP, a connectionless

protocol, can be connection-oriented. The point is that a TCP connection is virtual,

not physical. TCP operates at a higher level. TCP uses the services of IP to deliver individual

segments to the receiver, but it controls the connection itself. If a segment is lost

or corrupted, it is retransmitted. Unlike TCP, IP is unaware of this retransmission. If a

segment arrives out of order, TCP holds it until the missing segments arrive; IP is

unaware of this reordering.

In TCP, connection-oriented transmission requires three phases: connection establishment,

data transfer, and connection termination.

Connection Establishment

TCP transmits data in full-duplex mode. When two TCPs in two machines are connected,

they are able to send segments to each other simultaneously. This implies that

each party must initialize communication and get approval from the other party before

any data are transferred.

Three-Way Handshaking The connection establishment in TCP is called threeway

handshaking. In our example, an application program, called the client, wants to

make a connection with another application program, called the server, using TCP as

the transport layer protocol.

The process starts with the server. The server program tells its TCP that it is ready

to accept a connection. This is called a request for a passive open. Although the server

TCP is ready to accept any connection from any machine in the world, it cannot make

the connection itself.

The client program issues a request for an active open. A client that wishes to connect

to an open server tells its TCP that it needs to be connected to that particular

server. TCP can now start the three-way handshaking process as shown in Figure 23.18.

To show the process, we use two time lines: one at each site. Each segment has values

for all its header fields and perhaps for some of its option fields, too. However, we show

only the few fields necessary to understand each phase. We show the sequence number, the acknowledgment number, the control flags (only those that are set), and the window

size, if not empty. The three steps in this phase are as follows.

1. The client sends the first segment, a SYN segment, in which only the SYN flag is set.

This segment is for synchronization of sequence numbers. It consumes one sequence

number. When the data transfer starts, the sequence number is incremented by 1. We

can say that the SYN segment carries no real data, but we can think of it as containing

1 imaginary byte.

A SYN segment cannot carry data, but it consumes one sequence number.

2. The server sends the second segment, a SYN +ACK segment, with 2 flag bits set:

SYN and ACK. This segment has a dual purpose. It is a SYN segment for communication

in the other direction and serves as the acknowledgment for the SYN

segment. It consumes one sequence number.

A SYN +ACK segment cannot carry data,

but does consume one sequence number.

3. The client sends the third segment. This is just an ACK segment. It acknowledges

the receipt of the second segment with the ACK flag and acknowledgment number

field. Note that the sequence number in this segment is the same as the one in the

SYN segment; the ACK segment does not consume any sequence numbers.

An ACK segment, if carrying no data, consumes no sequence number.

Data Transfer

After connection is established, bidirectional data transfer can take place. The client

and server can both send data and acknowledgments. We will study the rules of

acknowledgment later in the chapter; for the moment, it is enough to know that data

traveling in the same direction as an acknowledgment are carried on the same segment.

The acknowledgment is piggybacked with the data. Figure 23.19 shows an example.

In this example, after connection is established (not shown in the figure), the client

sends 2000 bytes of data in two segments. The server then sends 2000 bytes in one segment.

The client sends one more segment. The first three segments carry both data and

acknowledgment, but the last segment carries only an acknowledgment because there

are no more data to be sent. Note the values of the sequence and acknowledgment

numbers. The data segments sent by the client have the PSH (push) flag set so that the

server TCP knows to deliver data to the server process as soon as they are received.

We discuss the use of this flag in greater detail later. The segment from the server, on

the other hand, does not set the push flag. Most TCP implementations have the option

to set or not set this flag.

Pushing Data We saw that the sending TCP uses a buffer to store the stream of data

coming from the sending application program. The sending TCP can select the segment

size. The receiving TCP also buffers the data when they arrive and delivers them to the

application program when the application program is ready or when it is convenient for

the receiving TCP. This type of flexibility increases the efficiency of TCP.

However, on occasion the application program has no need for this flexibility_ For

example, consider an application program that communicates interactively with another application program on the other end. The application program on one site wants to

send a keystroke to the application at the other site and receive an immediate response.

Delayed transmission and delayed delivery of data may not be acceptable by the application

program.

Connection Termination

Any of the two parties involved in exchanging data (client or server) can close the connection,

although it is usually initiated by the client. Most implementations today allow two

options for connection termination: three-way handshaking and four-way handshaking

with a half-close option.

Three-Way Handshaking Most implementations today allow three-way handshaking

for connection termination as shown in Figure 23.20.

1. In a normal situation, the client TCP, after receiving a close command from the

client process, sends the first segment, a FIN segment in which the FIN flag is set.

Note that a FIN segment can include the last chunk of data sent by the client, or it can be just a control segment as shown in Figure 23.20. If it is only a control segment,

it consumes only one sequence number.

The FIN segment consumes one sequence number ifit does not carry data.

2. The server TCP, after receiving the FIN segment, informs its process of the situation

and sends the second segment, a FIN +ACK segment, to confirm the receipt

of the FIN segment from the client and at the same time to announce the closing of

the connection in the other direction. This segment can also contain the last chunk

of data from the server. If it does not carry data, it consumes only one sequence

number.

The FIN +ACK segment consumes one sequence

number if it does not carry data.

3. The client TCP sends the last segment, an ACK segment, to confirm the receipt of

the FIN segment from the TCP server. This segment contains the acknowledgment

number, which is 1 plus the sequence number received in the FIN segment from

the server. This segment cannot carry data and consumes no sequence numbers.

Half-Close In TCP, one end can stop sending data while still receiving data. This is

called a half-close. Although either end can issue a half-close, it is normally initiated by

the client. It can occur when the server needs all the data before processing can begin. A

good example is sorting. When the client sends data to the server to be sorted, the server

needs to receive all the data before sorting can start. This means the client, after sending

all the data, can close the connection in the outbound direction. However, the inbound

direction must remain open to receive the sorted data. The server, after receiving the

data, still needs time for sorting; its outbound direction must remain open.

Figure 23.21 shows an example of a half-close. The client half-closes the connection

by sending a FIN segment. The server accepts the half-close by sending theACK segment.

The data transfer from the client to the server stops. The server, however, can still send

data. When the server has sent all the processed data, it sends a FIN segment, which is

acknowledged by an ACK from the client.

After half-closing of the connection, data can travel from the server to the client

and acknowledgments can travel from the client to the server. The client cannot send any

more data to the server. Note the sequence numbers we have used. The second segment

(ACK) consumes no sequence number. Although the client has received sequence number

y - 1 and is expecting y, the server sequence number is still y - 1. When the connection

finally closes, the sequence number of the last ACK segment is still x, because no

sequence numbers are consumed during data transfer in that direction.

congestion control in tcp

TCP uses a congestion window and a congestion policy that avoid congestion. Previously, we assumed that only the receiver can dictate the sender’s window size. We ignored another entity here, the network. If the network cannot deliver the data as fast as it is created by the sender, it must tell the sender to slow down. In other words, in addition to the receiver, the network is a second entity that determines the size of the sender’s window.

Congestion policy in TCP –

Slow Start Phase: starts slowly increment is exponential to threshold

Congestion Avoidance Phase: After reaching the threshold increment is by 1

Congestion Detection Phase: Sender goes back to Slow start phase or Congestion avoidance phase.

Slow Start Phase : exponential increment – In this phase after every RTT the congestion window size increments exponentially. In the slow start phase, the sender sets congestion window size = maximum segment size (1 MSS) at the initial stage. The sender increases the size of the congestion window by 1 MSS after receiving the ACK (acknowledgment). The size of the congestion window increases exponentially in this phase. The formula for determining the size of the congestion window is Congestion window size = Congestion window size + Maximum segment size.

Congestion Avoidance Phase

In this phase, after the threshold is reached, the size of the congestion window is increased by the sender linearly in order to avoid congestion. Each time an acknowledgment is received, the sender increments the size of the congestion window by 1.

Congestion Detection Phase : multiplicative decrement – If congestion occurs, the congestion window size is decreased. The only way a sender can guess that congestion has occurred is the need to retransmit a segment. Retransmission is needed to recover a missing packet that is assumed to have been dropped by a router due to congestion. Retransmission can occur in one of two cases: when the RTO timer times out or when three duplicate ACKs are received.

Case 1 : Retransmission due to Timeout – In this case congestion possibility is high.

(a) ssthresh is reduced to half of the current window size.

(b) set cwnd = 1

(c) start with slow start phase again.

Case 2 : Retransmission due to 3 Acknowledgement Duplicates – In this case congestion possibility is less.

(a) ssthresh value reduces to half of the current window size.

(b) set cwnd= ssthresh

(c) start with congestion avoidance phase

SSL/TLS protocol makes TCP a secure protocol, and whenever an application needs to send sensitive information over the internet, it is a requirement to use the send over SSL. often times the SSL protocol is used to secure — the application network layer — HTTP protocol. however SSL can be used to secure more data protocols the relay on TCP or a connection based protocol.

if you visit a page on your browser, and at the address bar shows https://..., this means that the HTTP protocol runs over secure TCP, or SSL/TLS, this also means that all data exchanged by your browser and the visited website are encrypted and no intruder or man in the middle can manipulate. this feels safe, doesn’t it.

The phases of an SSL connection

Here we have a simplified view of how SSL/TLS actually works to secure a TCP connection. an ssl connection runs over 3 phases; the handshake, key exchange and the actual data transfer. let’s take an example of a secure connection happening between a Client and a Server.

Beforehand, The SSL Certificate

A servier needs firest to aquire a valid SSL Certificate with a public key signed by a Certificate Authority (CA), this certificate is sent to any party wanting to connect to this server over SSL, and then that party checks with the CA weather this server is the real server that it needs to connect to. This is called Authenticating the server.

Phase 1: The handshake

The client makes a typical TCP connection to the server, by:

sending a SYN packet

recieving a SYN/ACK packet from the server

and finally sending an ACK packet to the server.

Phase 2: Exchanging keys

Then, after a TCP connection has established, comes the SSL part:

the client sends a hello to the server

the server sends back the singed certificate with the public key. it doesn’t send it’s private key.

the client checks the certificate wheather it’s valid.

and if the certificate looks valid, the client generates it’s own private key, encrypts it with the server’s public key, and sends this all back to the server.

the server unlooks the data with it’s own private key, gets the client private key.

the server uses the client’s private key to unlook the data part of the packets send by the client.

Phase 3: Data Transfer

Once a TCP connection is established, and the two parties able to encrypt and decrypt all transmitted data. The same proccess in last two steps in the second phase are repeated to send all segmented data over the created TCP connection using TCP segments.

These 3 phases mentioned above are just for demonstration and making things clear in the simplist way possible. However, a real SSL connection is way more complicated and has to handle data integrity with even an additional set of keys. This SSL/TLS protocol is updated with new encyption methods and to fix all security flaws that may arise.

You notice that I refer to the secure protocol using two abbriviations, SSL: Secure Socket Layer, and TLS: Tansport Layer Security. The thing is that SSL is actually the older version of this protocol and then TLS came (currently TLSv3) fixing many of the SSL problems. but for historical reasons only, both abbreviation refer to the same protocol.

QUALITY OF SERVICE

Reliability

Reliability is a characteristic that a flow needs. Lack of reliability means losing a

packet or acknowledgment, which entails retransmission. However, the sensitivity of

application programs to reliability is not the same. For example, it is more important

that electronic mail, file transfer, and Internet access have reliable transmissions than

telephony or audio conferencing.

Delay

Source-to-destination delay is another flow characteristic. Again applications can tolerate

delay in different degrees. In this case, telephony, audio conferencing, video conferencing,

and remote log-in need minimum delay, while delay in file transfer or e-mail is less

important.

Jitter

Jitter is the variation in delay for packets belonging to the same flow. For example, if

four packets depart at times 0, 1, 2, 3 and arrive at 20, 21, 22, 23, all have the same

delay, 20 units of time. On the other hand, if the above four packets arrive at 21, 23, 21,

and 28, they will have different delays: 21,22, 19, and 24.

For applications such as audio and video, the first case is completely acceptable;

the second case is not. For these applications, it does not matter if the packets arrive

with a short or long delay as long as the delay is the same for all packets. For this application,

the second case is not acceptable.

Jitter is defined as the variation in the packet delay. High jitter means the difference

between delays is large; low jitter means the variation is small.

Bandwidth

Different applications need different bandwidths. In video conferencing we need to

send millions of bits per second to refresh a color screen while the total number of bits

in an e-mail may not reach even a million.

What is DiffServ?

DiffServ is a model for providing QoS in the Internet by differentiating the traffic. The best effort method used in the internet tries to provide the best possible service depending on the varying traffic flow, rather than trying to differentiate the flow and provide higher level of service to some of the traffic. DiffServ tries to provide a improved level of service in the existing best effort environment by differentiating the traffic flow. For example, DiffServ will reduce the latency in traffic containing voice or streaming video, while providing best effort service to traffic containing file transfers. Packets are marked by the DiffServ devices at the boarders of the network with information about the level of service required by them. Other nodes in the network read this information and respond accordingly to provide the requested level of service.

What is IntServ?

IntServ is another model for providing QoS in networks. IntServ is based on building a virtual circuit in the internet using the bandwidth reservation technique. Requests for reserving the bandwidth come from the applications that require some kind of level of service. According to this model, each router in the network has to implement IntServ and each application that requires a service guarantee has to make a reservation. When bandwidth is reserved for a certain application, it cannot be reassigned for another application. Routers between the sender and the receiver determine whether they can support the reservation made by the application. If they cannot support it, they notify the receiver. Else they have to route the traffic to the receiver. Therefore, in this method, routers remember the properties of the traffic flow and also supervise it. The task of reserving paths would be very tedious in a busy network such as the Internet.

What is the difference between DiffServ and IntServ?

DiffServ is a model for providing QoS in the Internet by differentiating the traffic whereas IntServ is a model for providing QoS in networks by building a virtual circuit in the Internet using the bandwidth reservation technique. DiffServ does not require the nodes in the network to remember any state information about the flow as oppose to IntServ, which remembers state information in the routers. Furthermore, reserving paths and remembering state information in a busy network such as the Internet would be a tedious task. Therefore, implementing IntServ would be practically difficult in the Internet. Because of that, IntServ would be suitable for smaller private networks whereas DiffServ is much suitable for the Internet.